Guide Specification:  SOUND-ABATE™ Acoustical Shielding

Date: February 2018

Section 09 80 00 – Acoustic Treatment

Specifier Notes: This guide specification is written according to the Construction Specifications Institute (CSI) MasterFormat 2010. The section must be carefully reviewed and edited by the architect or engineer to meet the requirements of the project. Coordinate this section with other specification sections and the drawings.

Specifier Notes:  SOUND-ABATE™ acoustical shielding is a limp, high density vinyl sheet material, formulated from ethylene vinyl acetate (EVA); and designed for commercial, industrial and residential wall applications.  SOUND-ABATE™ is a high-quality, cost-effective solution to sound deadening and sound insulation needs. SOUND-ABATE™ will exceed your expectations for a quality building product, unmatched in cost-effectiveness and performance.

SOUND-ABATE™ is designed for installation behind gypsum drywall to block and dampen airborne sound transmission in homes and commercial buildings, where noise and sound transmission from room to room needs to be reduced.  SOUND-ABATE™ absorbs airborne sound vibrations that travel through gypsum drywall and dramatically reduces the movement of the sound to the other side.  SOUND-ABATE™ deadens sound transfer from shared walls, corridors, media rooms, workshops, laundries, etc.  SOUND-ABATE™ blocks water vapor transmission.

PART 1	 GENERAL

1.01 SECTION INCLUDES

- Furnishing and installation of acoustic sheet insulation for walls systems.

1.02 REFERENCES

- ASTM D3273 Standard Test Method for Resistance to Growth of Mold on the Surface of Interior Coatings in an Environmental Chamber.

- ASTM E84 Rev A Standard Test Method for Surface Burning Characteristics of Building Materials

- ASTM E90-02 Standard Specification for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions.

- ASTM E96 Standard Test Method for Water Vapor Transmission of Materials.

- ASTM E119-08 Fire Tests of Building Construction and Materials

- ASTM E413-87 Classification for Rating Sound Insulation.

- ASTM G21 Standard Practice for Determining Resistance of Synthetic Polymeric Materials to Fungi.

- ANSI/UL 263 Fire Tests of Building Construction and Materials.

- UL Classified for use in wall designs of the U300, U400, and V400 series.

1.03 SUBMITTALS

- Submit under provisions of Section 01 33 00.

- Product Data:  Manufacturer's catalog data and installation instructions for all conditions.

1.04 QUALITY ASSURANCE

- Manufacturer Qualifications:

  - All materials shall be supplied by a single manufacturer regularly engaged in manufacturing and supply of the product.

  - All materials shall be installed in accordance with manufacturer’s printed installation instructions.

- Installer Qualifications

  - Use an experienced installer and adequate number of skilled personnel who are thoroughly trained and experienced in work similar to that required for the project.

1.05 DELIVERY, STORAGE, AND HANDLING

- Deliver materials in manufacturer's original packages or bundles bearing the brand name of the product.

  - Inspect the materials upon delivery to assure that specified products have been received.

  - Report damaged material immediately to the delivering carrier and note such damage on the carrier's freight bill of lading.

- Storage and Handling

  - Store materials inside, level, under cover, in a well-ventilated area.

  - Keep material dry before installation, protect from weather and damage from construction operations and other causes.

  - Store at a location where humidity and temperature duplicate those during installation and occupancy.

PART 2	 PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:  BLUE RIDGE FIBERBOARD, 250 Celotex Dr., Danville, VA, 24541.

- Requests for substitutions will be considered in accordance with provisions of Section 01 25 00 – Substitution Procedures.

- Provide all limp, high density, EVA vinyl sheet from a single manufacturer.

2.02 MATERIALS

Specifier Note:  Consult manufacturer for specific recommendations of tested assembly 		constructions and STC ratings.

- Performance Based Specification:  Acoustical Shielding sheet shall be a limp, high density, ethylene vinyl acetate (EVA) formulated, vinyl sheet; physical properties as follows:

- Sheet size:  4’0” (1,219 mm) x 25’0” (7,620 mm)

- Thickness:  0.10” (2.54 mm)

- Weight: 1.0 lb/sf (nominal)

- Density: 120 lb/ft³ (1,922 kg/m³).

- Thermal Resistance (R- value):0.3 h·ft²·°F/Btu (0.23 K·m²/W) min per ASTM C518

- Sound Transmission Class (STC): 26 per ASTM E90

- Flame Spread Index: 140 FSI per ASTM E84 Rev A

- Smoke Developed Index: 250 SDI per ASTM E84

- Fire Resistance: One hour rated wall assembly per ASTM E119-08 and ANSI/UL 263

- Resistance to Mold Growth, ASTM D3273 and G21, highest resistance rating of 10, no visible growth.

- Water Vapor Transmission: < 0.50 perms, ASTM E96 Method A Desiccant

- Proprietary Based Specification

  - SOUND-ABATE Acoustical Shielding by BLUE RIDGE FIBERBOARD.

2.03	ACCESSORIES

Specifier Notes:  Installation accessories are dependent on substrates and installation methods as recommended by the manufacturer.  SOUND-ABATE must be installed behind gypsum drywall.

- Drywall:  ½” minimum thickness.

- Fasteners: Staples, nails or screws should be of sufficient size and length to hold materials in place until gypsum drywall can be installed.

PART 3 EXECUTION

3.01 EXAMINATION

- Examine substrates upon which work will be installed.  Notify Architect if surfaces are not acceptable. Do not begin surface preparation or application until unacceptable conditions have been corrected.

- Verify framing member spacing complies with manufacturer’s requirements depending on substrates and installation methods for different sound ratings.

- Verify environmental conditions are, and will continue to be, maintained in accordance 	with manufacturer’s recommendations.

- Coordinate with responsible entity to perform corrective work on unsatisfactory substrates or conditions.

- Starting work by installer is acceptance of substrate and environmental conditions.

3.02 INSTALLATION

Specifier Notes:  PRECAUTIONS:  Follow gypsum wallboard manufacturers installation recommendations or SOUND-ABATE manufacturer‘s recommendations for specific STC ratings.  SOUND-ABATE must not be used in close proximity to heat sources; chimneys, heater units, fireplaces, steam pipes, or other surfaces which could provide exposure to excessive heat (maximum 212° F) without adequate thermal protection. Consult the appropriate heating appliance manufacturer’s instructions before installation.  SOUNDA-ABATE should not be used as an exposed surface.  Gypsum drywall provides necessary fire resistance.  Where recessed lighting may be used, do not surround any light fixture due to possible high temperatures.  Where recessed lighting is used, lighting fixtures shall be UL certified, Type IC rated and installed according to the manufacturer instructions on clearance distance to combustible materials.

- Follow manufacturer’s written installation instructions.

  - Install acoustical shielding sheets in a clean, dry condition.

  - Ensure acoustical shielding sheets are thoroughly dry prior to closing in the structure.

  - Refer to manufacturer’s/supplier’s SDS data for these and other precautions.

- Utilize acoustical sealant, tape, sound rated putty and sound isolation materials as necessary to preserve the wall assembly’s acoustical integrity.

- Install acoustical shielding sheets onto wood or metal wall framing at 16” or 24” O.C.

- Align all panel ends and edges with framing members.

- Verify that appropriate framing section or drawing notes comply.

- Sheet Application:

  - Apply acoustical shielding sheets vertically to framing in accordance with manufacturer’s instructions.

Specifier Notes:  Follow gypsum wallboard manufacturers installation recommendations or SOUND-ABATE manufacturer‘s recommendations for specific STC ratings.

- Measure vertical wall height and cut the acoustical shielding sheet ½” short of this measurement.

- Roll the acoustical shielding sheet, position at top of wall, align vertical edges over center of studs, fasten along the sheet width at the top of the wall, and allow the sheet to unroll vertically.

- Butt vertical edges together and apply fasteners along vertical edges as needed.

- Ensure vertical seams center over studs. Trim sheet if necessary.

- Locate any wall outlet penetrations that require access and cut an “X” access point for each.

- Repeat the previous procedure for each additional sheet to be installed.

- Utilize acoustical sealant, tape, sound rated putty and sound isolation materials as necessary to preserve the wall assembly’s acoustical integrity.

- Cover with gypsum drywall, ensuring joints of drywall and acoustical shielding sheets are staggered.

3.04 ADJUSTING AND CLEANING

- Follow manufacturer's instructions for replacement of damaged sheets.

- Remove excess materials, equipment and debris; and dispose of properly away from the premises during the course of the work and on completion of the work.

- Leave work in clean condition in accordance with Section 01 50 00 – Temporary Facilities and Controls.

3.5 PROTECTION

- Install gypsum drywall finish after application of the acoustic sheeting as soon as 	practical.

END OF SECTION